# COMPUTER VISION – ASSESSMENT TOOL 3 (MCQ)

- Image Normalization (Preprocessing)

An image pixel value r = 150 is normalized to the range [0,1] from 8-bit data.

  - 0.39

  - 0.49

  - 0.59

  - 0.78

Answer: B) 0.49

- Image Storage Calculation

A grayscale image of size 1024 × 512 is stored using 8 bits per pixel. What is the storage requirement? A) 256 KB

- 512 KB

- 1024 KB

- 2048 KB

Answer: B) 512 KB

- Edge Detection – Gradient Magnitude

Given Sobel gradients: Gx = 6, Gy = 8. What is the gradient magnitude?

  - 10

  - 12

  - 14

  - 16

Answer: A) 10

- Edge Direction Calculation

For the same gradients (Gx = 6, Gy = 8), what is the edge direction?

  - 36.8°

  - 45°

  - 53.1°

  - 60°

Answer: C) 53.1°

- Harris Corner Detection Response

Given det(M) = 200, trace(M) = 30, k = 0.04. What is the corner response R?

  - 136

  - 164

  - 180

  - 200

Answer: A) 164

- Harris Corner Classification

If the Harris response R = +250, the pixel is classified as:

  - Flat region

  - Edge

  - Corner

  - Noise

Answer: C) Corner

- Hough Transform – Line Representation

A line detected at θ = 90°and ρ = 50 corresponds to:

  - Horizontal line at y = 50

  - Vertical line at x = 50

  - Diagonal line

  - Random line

Answer: B) Vertical line at x = 50

- SIFT Scale

An image is scaled by a factor of 2. If original keypoint scale is σ = 1.6, new scale is:

  - 0.8

  - 1.6

  - 3.2

  - 4.0

Answer: C) 3.2

- PCA Dimensionality Reduction

A feature vector has 100 dimensions. PCA retains 95% variance using 20 components. What is the dimensionality after PCA? A) 5

- 20

- 80

- 95

Answer: B) 20

- PCA Covariance Matrix Size

For a dataset with 50 features, what is the size of the covariance matrix?

  - 50 × 1

  - 1 × 50

  - 50 × 50

  - 100 × 100

Answer: C) 50 × 50

- Image Quantization (Preprocessing)

A grayscale image is quantized from 256 levels to 32 levels. What is the number of bits per pixel after quantization? A) 3

B) 4

C) 5

D) 6

Answer: C) 5

- Image Storage Reduction

An RGB image of size 512 × 512 uses 8 bits per channel. It is converted to grayscale with 8 bits per pixel. What is the percentage reduction in storage?

  - 33%

  - 50%

  - 66.7%

  - 75%

Answer: C) 66.7%

- Sobel Edge Response

Given Sobel kernel response values: Gx = -12, Gy = 5. What is the gradient magnitude (approx.)? A) 10

- 12

- 13

- 15

Answer: C) 13

- Gradient Direction Calculation

For Gx = -12, Gy = 5, the gradient direction is closest to:

  - 22°

  - 157°

  - 180°

  - −22°

Answer: B) 157°

- Harris Corner Detection Response

Given det(M) = 400, trace(M) = 40, k = 0.06. Compute the Harris response R.

  - 256

  - 304

  - 352

  - 400

Answer: A) 304

- Harris Corner Decision

For a pixel, eigenvalues of matrix M are λ₁ = 120, λ₂ = 130. This pixel is classified as: A) Flat region

- Edge

- Corner

- Noise

Answer: C) Corner

- Hough Transform Voting

A point contributes votes to 180 values of θ in Hough space. What is the shape of its representation? A) A point

- A straight line

- A sinusoidal curve

- A circle

Answer: C) A sinusoidal curve

- Hough Line Detection

A detected line has parameters ρ = 0, θ = 45°. Which line is represented?

  - Horizontal through origin

  - Vertical through origin

  - Diagonal line through origin

  - Random orientation

Answer: C) Diagonal line through origin

- SIFT Keypoint Descriptor Size

Each SIFT descriptor consists of 4×4 histograms, each with 8 bins. What is the descriptor dimensionality? A) 64

- 96

- 128

- 256

Answer: C) 128

- PCA Eigenvalue Selection

Eigenvalues of covariance matrix: [40, 25, 15, 10, 5, 5]. How many principal components are needed to retain 80% variance? A) 2

B) 3

C) 4

D) 5

Answer: B) 3

- Preprocessing – Normalization Scenario

A surveillance camera outputs 8-bit grayscale images. Before feature extraction, pixel values must be normalized to [0,1]. A pixel has value 204. What is the normalized value?

  - 0.60

  - 0.70

  - 0.80

  - 0.90

Answer: C) 0.80

- Image Representation Scenario

A medical image of size 2048 × 1024 is stored using 16 bits per pixel. What is the memory required?

  - 2 MB

  - 4 MB

  - 8 MB

  - 16 MB

Answer: B) 4 MB

- Edge Detection Scenario

At a pixel location, Sobel gradients are Gx = 9, Gy = 12. What is the edge magnitude at that pixel?

  - 15

  - 18

  - 21

  - 25

Answer: A) 15

- Edge Orientation Scenario

Using the same gradients (Gx = 9, Gy = 12), what is the edge direction (approx.)?

  - 36.8°

  - 45°

  - 53.1°

  - 60°

Answer: C) 53.1°

- Harris Corner Detection Scenario

For a region in an image, det(M) = 300, trace(M) = 25, k = 0.04. What is the Harris response R?

  - 200

  - 250

  - 275

  - 300

Answer: B) 275

- Harris Corner Interpretation Scenario

If the Harris response R is large and positive, the pixel corresponds to:

  - Flat region

  - Edge

  - Corner

  - Noise

Answer: C) Corner

- Hough Transform Scenario

A point in image space votes for all possible θ values in Hough space. What shape does this vote form?

  - A point

  - A straight line

  - A sinusoidal curve

  - A circle

Answer: C) A sinusoidal curve

- Hough Line Detection Scenario

A detected line has parameters ρ = 40, θ = 90°. Which line is represented?

  - Vertical line x = 40

  - Horizontal line y = 40

  - Diagonal line

  - Random orientation Answer: A) Vertical line x = 40

- SIFT Feature Scenario

An image is scaled by a factor of 0.5. A SIFT keypoint detected at scale σ = 2.0 originally will now appear at: A) 1.0

- 0.5

- 2.0

- 4.0

Answer: A) 1.0

- PCA Dimensionality Reduction Scenario

A feature vector has 200 dimensions. After PCA, 30 principal components retain 92% variance. What is the new feature dimension? A) 30

- 92

- 170

- 200

Answer: A) 30

- Preprocessing – Bit Depth Conversion

A grayscale image originally stored in 12-bit format is converted to 8-bit before processing. An original pixel value is 3000. What is the approximate 8-bit value after linear scaling?

  - 120

  - 150

  - 187

  - 225

Answer: C) 187

- Image Representation – Sampling Scenario

A camera samples an image at 600 × 400 pixels. The image is down-sampled by a factor of 2 in both directions. What is the new image size?

  - 150 × 100

  - 300 × 200

  - 400 × 300

  - 600 × 400

Answer: B) 300 × 200

- Edge Detection – Sobel Response

At a pixel, Sobel gradients are Gx = -8, Gy = 15. What is the edge magnitude (approx.)? A) 13

- 15

- 17

- 20

Answer: C) 17

- Edge Orientation Scenario

For Gx = -8, Gy = 15, the gradient direction is closest to:

  - 28°

  - 62°

  - 118°

  - 152°

Answer: C) 118°

- Harris Corner Detection – Computation

Given det(M) = 500, trace(M) = 35, k = 0.05. What is the Harris response R?

  - 439

  - 450

  - 469

Answer: A) 439

- Harris Corner Classification

If one eigenvalue is large and the other is small, the pixel represents:

  - Flat region

  - Edge

  - Corner

  - Noise Answer: B) Edge

- Hough Transform – Line Interpretation

A line detected with parameters ρ = 0, θ = 90°corresponds to:

  - Vertical line at x = 0

  - Horizontal line at y = 0

  - Diagonal line

  - Random orientation Answer: A) Vertical line at x = 0

- SIFT Descriptor Scenario

Each SIFT descriptor contains 4 × 4 blocks with 8 orientation bins each. How many values describe one keypoint? A) 32

- 64

- 96

- 128

Answer: D) 128

- PCA Variance Retention

Eigenvalues of covariance matrix are [50, 30, 10, 5, 5]. How many principal components are needed

to retain 90% variance? A) 1

B) 2

C) 3

D) 4

Answer: C) 3

- PCA Projection Scenario

Original feature vector dimension = 120. PCA reduces it to 25 components. What is the reduction percentage? A) 60%

- 70%

- 79%

- 80%

Answer: C) 79%

- Quantization Error Analysis (Preprocessing)

An 8-bit grayscale image is quantized to 64 levels. Original pixel value = 150. What is the maximum possible quantization error? A) ±1

- ±2

- ±4

- ±8

Answer: B) ±2

- Image Representation Storage Comparison

Image A: 1024 × 1024 grayscale (8-bit). Image B: 1024 × 1024 RGB (8-bit per channel). What is the storage ratio (B : A)? A) 1 : 1

- 2 : 1

- 3 : 1

- 4 : 1

Answer: C) 3 : 1

- Sobel Edge Strength Comparison

At two pixels: Pixel-1: Gx = 6, Gy = 8. Pixel-2: Gx = 9, Gy = 12. Which pixel corresponds to a stronger edge? A) Pixel-1

- Pixel-2

- Both equal

- Neither Answer: B) Pixel-2

- Harris Corner Response Comparison

Two regions: R1 det(M)=300, trace(M)=25; R2 det(M)=350, trace(M)=40. Given k=0.04, which region

is more likely a corner? A) R1

- R2

- Both

- Neither Answer: B) R2

- Harris Corner vs Edge Decision

Eigenvalues of matrix M at a pixel are λ₁ = 120, λ₂ = 10. What does this pixel represent?

  - Flat region

  - Edge

  - Corner

  - Noise Answer: B) Edge

- Hough Transform Voting Analysis

Two points generate curves in Hough space that intersect at (ρ = 60, θ = 45°). What does this intersection indicate? A) A corner

- A circle

- A line in image space

- Noise accumulation

Answer: C) A line in image space

- Hough Transform Parameter Effect

Increasing the θ resolution from 1° to 0.5°results in:

  - Fewer accumulator cells

  - Lower memory usage

  - Higher line detection accuracy Answer: C) Higher line detection accuracy

- SIFT Scale Analysis

An object appears at scale σ = 1.6 in Image-1. In Image-2, the object is scaled by 1.5×. At what scale should the corresponding SIFT keypoint appear?

  - 1.1

  - 1.6

  - 2.4

  - 3.1

Answer: C) 2.4

- PCA Variance Contribution Analysis

Eigenvalues of covariance matrix are [60, 20, 10, 5, 5]. How many principal components are needed

to retain 90% variance? A) 1

B) 2

C) 3

D) 4

Answer: C) 3

- PCA Dimensionality Reduction Impact

Original feature dimension = 150. After PCA = 30 components. What is the percentage reduction?

  - 60%

  - 70%

  - 80%

  - 85%

Answer: C) 80%